## H13	STRUCTURAL GLASS WALLING

TYPE OF STRUCTURAL GLASS ASSEMBLY

### H13	ELLIS PEARSON GLASS-BACK WALLS FOR SQUASH COURTS

Manufactured and supplied by Prospec Limited, Canklow Meadows Estate, West Bawtry Road, Rotherham S60 2XL (Tel. 01709 377147) and as approved by the WSF and England Squash.

Type 200; 4 panel freestanding glasswall; overall size 6400mm wide x 2130mm high

Constructed and installed in accordance with Prospec specification and working drawings from 12mm toughened glass, edgeworked and drilled as necessary.

Manifestation to the lower 610mm of all panels and doors with 3mm wide white lines screen painted on the playing side.

Include all components and high tensile patch fittings, flush ring pull handles and latch set, supporting glass fins 300mm wide and aluminium brackets, glazing channels, silicone glazing compound, all screws, bushes, fixing bolts etc. required to complete the installation.